ECPI University

CIS_202L

By: Rafat Khandaker

Lab 3

Instructor:

Albert Ostering

11/26/18

LAB 1:                          (Pings from PC A into PC-C & Lo0)

a. Test connectivity by pinging from each PC to the default gateway that has been configured for that host.

From PC-A, is it possible to ping the default gateway? YES

From PC-C, is it possible to ping the default gateway? YES

b. Test connectivity by pinging between the directly connected routers.

From R1, is it possible to ping the S0/0/0 interface of R3? YES

**If the answer is no to any of these questions, troubleshoot the configurations and correct the error**

c. Test connectivity between devices that are not directly connected.

From PC-A, is it possible to ping PC-C? YES

From PC-A, is it possible to ping Lo0? YES

From PC-A, is it possible to ping Lo1? YES

Were these pings successful? Why or why not?

The pings are successful because the default route for router 1 is router 2 & vice versa.  When a packet with a destination address is unknown to router 1, it will be forwarded to router 2. Lo0, Lo1 & PC –C is contained in Router 2 LAN network, so router 2 will be able to identify the destination address on its local area network.

Show IP Int Brief

Router 1

Router 3

a. Check the status of the interfaces on R1 with the show IP interface brief command.

How many interfaces are activated on R1?  2 Interfaces

b. Check the status of the interfaces on R3.

How many interfaces are activated on R3?  4 Interfaces

c. View the routing table information for R1 using the show IP route command.

Show IP Route on R1

What networks are present in the Addressing Table of this lab, but not in the routing table for R1?

    - Router 1 Only contain routes from Its Internal LAN & The Directly Connected Router 3 IP Address. It does not contain, The network for Lo0, Lo1 & Network for PC-C.

d. View the routing table information for R3.

What networks are present in the Addressing Table in this lab, but not in the routing table for R3?

    - Router 3 contain all the Lo0, Lo1, Internal Lan and directly connected route through its Serial link. It does not contain the route table for PC-A Network:

( 192.168.0.0/24 )

Why are all the networks not in the routing tables for each of the routers?

  - There was no static route or routing protocol used in the configuration for the routing table. Instead a default route was set to each router, this will forward all packets outside the routing table to the adjacent router.

a. On the R1 router, configure a static route to the 192.168.1.0 network using the IP address of the Serial

0/0/0 interface of R3 as the next-hop address. Write the command you used in the space provided.

    - IP Route 192.168.x.x 255.255.255.0 10.1.1.x (on each router)

b. View the routing table to verify the new static route entry.

How is this new route listed in the routing table?

  - When configured these routes will show up as static configured: ‘S’ code.

From host PC-A, is it possible to ping the host PC-C?

  - Yes, Previously I configured with a default route, which allowed devices to ping each other. Static route should allow devices to ping each other. In this case it is best to create a static single route with no default route, because default routing into an unknown network by both routers will create congestion within the serial link.

a. On the R3 router, configure a static route to the 192.168.0.0 network using S0/0/0 as the exit interface.

Write the command you used in the space provided.

  - Ip route 192.168.x.x 255.255.255.0 Serial2/0

(in this case I used S2/0 on both routers)

b. View the routing table to verify the new static route entry.

How is this new route listed in the routing table?

  - The new routes are listed as a Static route, by ‘S’ code via serial link.

c. From host PC-A, is it possible to ping the host PC-C?

  - YES

Step 3:                                (Configure a static route)

a. On the R1 router, configure a static route to the 198.133.219.0 network using one of the static routes:

configuration options from the previous steps. Write the command you used in the space provided.

  - ip route 192.133.219.0 255.255.255.0 Serial2/0

b. On the R1 router, configure a static route to the 209.165.200.224 network on R3 using the other static

route configuration option from the previous steps. Write the command you used in the space provided.

  - ip route 209.165.200.224 255.255.255.224 Serial2/0

c. View the routing table to verify the new static route entry.

How is this new route listed in the routing table?

  - The new routes are listed as a Static route, denoted by ‘S’ code.

d. From host PC-A, is it possible to ping the R1 address 198.133.219.1?

This ping should be successful.

  - Yes, it should.

On R1, use the no command to remove the static routes for the two loopback addresses from the routing

table. Write the commands you used in the space provided.

- no ip route 192.133.219.0 255.255.255.0 Serial2/0

- no ip route 209.165.200.224 255.255.255.224 Serial2/0

- no ip route 192.168.x.x 255.255.255.0 Serial2/0

b. View the routing table to verify the routes have been removed.

How many network routes are listed in the routing table on R1?

  - Only 2 routes should be connected

Is the Gateway of last resort set?

    - YES

What is the Gateway of last resort? 10.1.1.2

c. From host PC-A, is it possible to ping the 209.165.200.225?  YES

d. From host PC-A, is it possible to ping the 198.133.219.1?  YES

Reflection

1. A new network 192.168.3.0/24 is connected to interface fa0/0 on R1. What commands could be used to

configure a static route to that network from R3?

  - Ip route 192.168.3.0 255.255.255.0 fa0/0

2. Is there a benefit to configuring a directly connected static route instead of a recursive static route?

  - The benefit is that all request to that static network will only go out from that interface, configured. This means that no two interfaces can route the same network.

3. Why is it important to configure a default route on a router?

  - The benefit of a default route is the ability to configure with redundancy. If I have only a single cable towards a core router, for example, then there is no point in statically routing all the networks on that router. We might as well just set a default route into the core router which will have the decision to make network-based routing decisions.

LAB 2    (Reflection)

1. Why should you configure the vty lines for the switch?

- The VTY line is configured for telnet access to the switch, to access remotely.

2. Why change the default VLAN 1 to a different VLAN number?

- VLAN 1 is default & is sent through the default Native VLAN trunk. Which means that by default VLAN 1 can be shared through all Trucking cables. It is considered bad security practice to do so. Also because VLAN 1 is used for VLAN management.

3. How can you prevent passwords from being sent in plain text?

- You can configure service password-encryption command.

4. Why configure a static MAC address on a port interface?

  - Mac sticky will ensure that only one device with the correct hardware address can access the port & will be in shutdown for all other mac address. It is a good practice security configuration.

LAB 3

Can PC-A ping PC-B?

Can PC-A ping PC-C?

Can PC-A ping S1?

Can PC-B ping PC-C?

Can PC-B ping S2?

Can PC-C ping S2?

YES !

What is the default VLAN?

- Default VLAN is configured at VLAN 1

What ports are assigned to the default VLAN?

- All ports, by default are configured at VLAN 1

What is the status of VLAN 99? Why?

- It is active, once the VLAN 99 is created, it is activated.

Why might you want to manually configure an interface to trunk mode instead of using DTP?

- By Manually configuring trunk ports, we disable dynamic updates of VLAN information between trunk ports. This can be a security hazard if configured and is considered bad practice.  DTP is a cisco proprietary protocol that is enabled by default.

To initialize a switch back to its default settings, what other commands are needed?

- “write erase”,  ‘Erase Startup Config’

Reflection

1. What is needed to allow hosts on VLAN 10 to communicate to hosts on VLAN 20?

- Routing needs to be configured to enable VLAN 10 to communicate with VLAN 20

2. What are some primary benefits that an organization can receive through effective use of VLANs?

- This enhance the security of department, better management of IP addressing through subnetting. Ensures to limit access to resources, “principal of least privilege.”